Armierungspläne TF without rigid foam in the socket brim

Inhalt

4R41, 4R42, 4R43, 4R89, 4R111, 4R116, 4R119, 6A20=10;30 up to 100 kg:	2

4R41, 4R42, 4R43, 4R89, 4R111, 4R116, 4R119, 6A20=10;30 up to 125 kg:	3

6A30=10, 6A30=20 up to 100 kg	4

6A30=10, 6A30=20 up to 125kg	5

5R1 up to 100 kg	6

5R1 up to 125 kg	7

5R2, 5R2=C up to 100 kg	8

5R2, 5R2=C up to 125 kg	9

SF5R, SF6A up to 100 kg	10

SF5R, SF6A up to 125 kg	11

# 4R41, 4R42, 4R43, 4R89, 4R111, 4R116, 4R119, 6A20=10; 30 up to 100 kg:

| Preparation Step |  |  |  |
| --- | --- | --- | --- |
| Material | Kenn-zeichen | Lagen Anzahl | Kommentar |
| Dakron Felt | 616G6=* | 1,5 | 1x completely and 2nd layer proximal socket brim |
| Perlon Stockinette | 623T3=* | 2 | completely |
| Carbon Fiber webbing - unidirectional | 616B1=50x | 1 | width 5 cm circular 3 cm below perineum |
| Carbon Fiber webbing - unidirectional | 616B1=100x | 1 | Lateral - Distal |
| Carbon Fiber webbing - unidirectional | 616B1=50x | 2 | as V in ischial tuberosity seat |
| Carbon Fiber Cloth | 616G12=* | 3 | 15x15 cm distal stump end slightly shifted |
| Perlon Stockinette | 623T3=* | 2 | completely |
| Final Step |  |  |  |
| Adapter |  |  | With Siegelharz / Talkum 1:1 fixing the Adaptor and make a form-closed shape |
| Perlon Stockinette | 623T3=* | 1 | completely |
| Carbon Fiber Cloth | 616G12=* | 1 | Strips over anchor arms |
| Perlon Stockinette | 623T3=* | 1 | completely |
| Woven Carbon Fiber Stockinette | 616G15=* | 1,5 | First layer up to the circular carbon webbing 2nd layer over anchor arms |
| Perlon Stockinette | 623T3=* | 2 | completely |

# 4R41, 4R42, 4R43, 4R89, 4R111, 4R116, 4R119, 6A20=10; 30 up to 125 kg:

| Preparation Step |  |  |  |  |
| --- | --- | --- | --- | --- |
| Material | Kenn-zeichen |  | Lagen Anzahl | Kommentar |
| Dakron Felt | 616G6=* |  | 1,5 | 1x completely and 2nd layer proximal socket brim |
| Perlon Stockinette | 623T3=* |  | 2 | completely |
| Carbon Fiber webbing - unidirectional | 616B1=50x |  | 2 | width 5 cm circular 3 cm below perineum |
| Carbon Fiber webbing - unidirectional | 616B1=50x |  | 1 | 5 cm breit Medial-Distal-Lateral |
| Carbon Fiber webbing - unidirectional | 616B1=100x |  | 1 | Lateral - Distal |
| Carbon Fiber webbing - unidirectional | 616B1=50x |  | 4 | as V in ischial tuberosity seat |
| Carbon Fiber Cloth | 616G12=* |  | 3 | 15x15 cm distal stump end slightly shifted |
| Perlon Stockinette | 623T3=* |  | 2 | completely |
| Final Step |  |  |  |  |
| Adapter |  |  |  | With Siegelharz / Talkum 1:1 fixing the Adaptor and make a form-closed shape |
| Perlon Stockinette | 623T3=* | 1 |  | completely |
| Carbon Fiber Cloth | 616G12=* | 2 |  | Strips over anchor arms |
| Perlon Stockinette | 623T3=* | 1 |  | completely |
| Woven Carbon Fiber Stockinette | 616G15=* | 2 |  | Both layers up to the circular carbon webbing |
| Perlon Stockinette | 623T3=* | 2 |  | completely |

# 6A30=10, 6A30=20 up to 100 kg

| Preparation Step |  |  |  |
| --- | --- | --- | --- |
| Material | Kenn-zeichen | Anzahl Lagen | Kommentar |
| Dummy |  |  | from 6A30=* |
| PVA Folie | 99B71=* |  |  |
| Shuttle Lock | 6A30=* | 1 | Fixing shuttle lock on the dummy |
| Dakron Felt | 616G6=* | 1,5 | 1x completely and 2nd layer proximal socket brim |
| Perlon Stockinette | 99B71=* | 2 | completely |
| Carbon Fiber webbing - unidirectional | 616B1=50x | 1 | width 5 cm circular 3 cm below perineum |
| Carbon Fiber webbing - unidirectional | 616B1=100x | 1 | Lateral - Distal |
| Carbon Fiber webbing - unidirectional | 616B1=50x | 2 | as V in ischial tuberosity seat |
| Carbon Fiber Cloth | 616G12=* | 3 | 15x15 cm distal stump end slightly shifted |
| Perlon Stockinette | 623T3=* | 2 | completely |
| Final Step |  |  |  |
| Choos the lamination plan for Final Step according to the Socket adaptor. 5R1 up to 100 kg page 6 5R2 up to 100 kg page 8 |  |  |  |

# 6A30=10, 6A30=20 up to 125kg

| Preparation Step |  |  |  |
| --- | --- | --- | --- |
| Material | Kenn-zeichen | Lagen Anzahl | Kommentar |
| Dummy |  |  | von 6A30=* |
| PVA Folie |  |  |  |
| Shuttle |  | 1 | aufsetzen |
| Dakron Felt | 616G6=* | 1 | 1x completely and 2nd layer proximal socket brim |
| Perlon Stockinette | 623T3=* | 2 | completely |
| Carbon Fiber webbing - unidirectional. | 616B1=50x | 2 | width 5 cm circular 3 cm below perineum |
| Carbon Fiber webbing - unidirectional | 616B1=100x | 1 | Lateral - Distal |
| Carbon Fiber webbing - unidirectional | 616B1=50x | 1 | 5 cm breit Medial-Distal-Lateral |
| Carbon Fiber webbing - unidirectional | 616B1=50x | 4 | as V in ischial tuberosity seat |
| Carbon Fiber Cloth | 616G12=* | 3 | 15x15 cm distal stump end slightly shifted |
| Perlon Stockinette | 623T3=* | 2 | completely |
| Final Step |  |  |  |
| Choos the lamination plan for Final Step according to the Socket adaptor. 5R1 up to 100 kg page 7 5R2 up to 100 kg page 9 |  |  |  |

# 5R1 up to 100 kg

| Preparation Step |  |  |  |  |
| --- | --- | --- | --- | --- |
| Material | Kenn-zeichen |  | Lagen Anzahl | Kommentar |
| Dakron Felt | 616G6=* |  | 1,5 | 1x completely and 2nd layer proximal socket brim |
| Perlon Stockinette | 623T3=* |  | 2 | completely |
| Carbon Fiber webbing - unidirectional | 616B1=50x |  | 1 | width 5 cm circular 3 cm below perineum |
| Carbon Fiber webbing - unidirectional | 616B1=100x |  | 1 | Lateral - Distal |
| Carbon Fiber webbing - unidirectional | 616B1=50x |  | 2 | as V in ischial tuberosity seat |
| Carbon Fiber Cloth | 616G12=* |  | 3 | 15x15 cm distal stump end slightly shifted |
| Perlon Stockinette | 623T3=* |  | 2 | completely |
| Final Step |  |  |  |  |
| Adapter |  |  |  | With Siegelharz / Talkum 1:1 or rigid foam fixing the Adapter and make a form-closed shape |
| Perlon Stockinette | 623T3=* | 1 |  | completely |
| Carbon Fiber webbing - unidirectional | 616B1=50x | 1 |  | 5 cm breit, Medial-Distal-Lateral und Anterior-Distal-Posterior bis Schaftmitte |
| Perlon Stockinette | 623T3=* | 1 |  | completely |
| Woven Carbon Fiber Stockinette | 616G15=* | 1,5 |  | First layer up to the circular carbon webbing 2nd layer over anchor arms |
| Perlon Stockinette | 623T3=* | 2 |  | completely |

# 5R1 up to 125 kg

| Preparation Step |  |  |  |  |
| --- | --- | --- | --- | --- |
| Material | Kenn-zeichen | Lagen Anzahl |  | Kommentar |
| Dakron Felt | 616G6=* | 1,5 |  | 1x completely and 2nd layer proximal socket brim |
| Perlon Stockinette | 623T3=* | 2 |  | completely |
| Carbon Fiber webbing - unidirectional | 616B1=50x | 2 |  | width 5 cm circular 3 cm below perineum |
| Carbon Fiber webbing - unidirectional | 616B1=100x | 1 |  | Lateral - Distal |
| Carbon Fiber webbing - unidirectional | 616B1=50x | 1 |  | 5 cm breit Medial-Distal-Lateral |
| Carbon Fiber webbing - unidirectional | 616B1=50x | 4 |  | as V in ischial tuberosity seat |
| Carbon Fiber Cloth | 616G12=* | 3 |  | 15x15 cm distal stump end slightly shifted |
| Perlon Stockinette | 623T3=* | 2 |  | completely |
| Final Step |  |  |  |  |
| Adapter |  |  | With Siegelharz / Talkum 1:1 or rigid foam fixing the Adapter and make a form-closed shape |  |
| Perlon Stockinette | 623T3=* | 1 | completely |  |
| Carbon Fiber webbing - unidirectional | 616B1=50x | 2 | 5 cm breit, Medial-Distal-Lateral und Anterior-Distal-Posterior bis Schaftmitte |  |
| Perlon Stockinette | 623T3=* | 1 | completely |  |
| Woven Carbon Fiber Stockinette | 616G15=* | 2 | First layer up to the circular carbon webbing 2nd layer over anchor arms |  |
| Perlon Stockinette | 623T3=* | 2 | completely |  |

# 5R2, 5R2=C up to 100 kg

| Preparation Step |  |  |  |  |
| --- | --- | --- | --- | --- |
| Material | Kenn-zeichen |  | Lagen Anzahl | Kommentar |
| Dakron Felt | 616G6=* |  | 1,5 | 1x completely and 2nd layer proximal socket brim |
| Perlon Stockinette | 623T3=* |  | 2 | completely |
| Carbon Fiber webbing - unidirectional | 616B1=50x |  | 1 | width 5 cm circular 3 cm below perineum |
| Carbon Fiber webbing - unidirectional | 616B1=100x |  | 1 | Lateral - Distal |
| Carbon Fiber webbing - unidirectional | 616B1=50x |  | 2 | as V in ischial tuberosity seat |
| Carbon Fiber Cloth | 616G12=* |  | 3 | 15x15 cm distal stump end slightly shifted |
| Perlon Stockinette | 623T3=* |  | 2 | completely |
| Final Step |  |  |  |  |
| Adapter |  |  |  | With Siegelharz / Talkum 1:1 or rigid foam fixing the Adapter and make a form-closed shape |
| Perlon Stockinette | 623T3=* | 1 |  | completely |
| Carbon unidirectional Stockinette | 616G2=* | 1,5 |  | Erste Lage komplett, zweite bis Schaftmitte |
| Perlon Stockinette | 623T3=* | 1 |  | completely |
| Woven Carbon Fiber Stockinette | 616G15=* | 1,5 |  | First layer up to the circular carbon webbing 2nd layer over anchor arms |
| Perlon Stockinette | 623T3=* | 2 |  | completely |

# 5R2, 5R2=C up to 125 kg

| Preparation Step |  |  |  |
| --- | --- | --- | --- |
| Material | Kenn-zeichen | Lagen Anzahl | Kommentar |
| Dakron Felt | 616G6=* | 1,5 | 1x completely and 2nd layer proximal socket brim |
| Perlon Stockinette | 623T3=* | 2 | completely |
| Carbon Fiber webbing - unidirectional | 616B1=50x | 2 | width 5 cm circular 3 cm below perineum |
| Carbon Fiber webbing - unidirectional | 616B1=100x | 1 | Lateral - Distal |
| Carbon Fiber webbing - unidirectional | 616B1=50x | 1 | 5 cm breit Medial-Distal-Lateral |
| Carbon Fiber webbing - unidirectional | 616B1=50x | 4 | as V in ischial tuberosity seat |
| Carbon Fiber Cloth | 616G12=* | 3 | 15x15 cm distal stump end slightly shifted |
| Perlon Stockinette | 623T3=* | 2 | completely |
| Final Step |  |  |  |
| Adapter |  |  | With Siegelharz / Talkum 1:1 or rigid foam fixing the Adapter and make a form-closed shape |
| Perlon Stockinette | 623T3=* | 1 | completely |
| Carbon unidirectional Stockinette | 616G2=* | 1,5 | Erste Lage komplett, zweite bis Schaftmitte |
| Perlon Stockinette | 623T3=* | 1 | completely |
| Woven Carbon Fiber Stockinette | 616G15=* | 2 | First layer up to the circular carbon webbing 2nd layer over anchor arms |
| Perlon Stockinette | 623T3=* | 2 | completely |

# SF5R, SF6A up to 100 kg

| Preparation Step |  |  |  |
| --- | --- | --- | --- |
| Material | Kenn-zeichen | Lagen Anzahl | Kommentar |
| Adapter |  |  | aufsetzen |
| Dakron Felt | 616G6=* | 1,5 | 1x completely and 2nd layer proximal socket brim |
| Carbon Fiber webbing - unidirectional | 616B1=50X | 1 | width 5 cm circular 3 cm below perineum |
| Woven Carbon Fiber Stockinette | 616G15=* | 1,5 | Completely, cut along the socket brim 2nd layer up to the circular carbon webbing |
| Carbon Fiber webbing - unidirectional | 616B1=50X | 2 | as V in ischial tuberosity seat |
| Carbon Fiber Cloth | 616G12=* | 3 | 15x15 cm distal stump end slightly shifted |
| Perlon Stockinette | 623T3=* | 2 | completely |
| Final Step |  |  |  |
| Perlon Stockinette | 623T3=* | 1 | completely |
| Carbon Fiber Cloth | 616G12=* | 1 | 15x15 cm distal stump end slightly shifted |
| Woven Carbon Fiber Stockinette | 616G15=* | 1,5 | First layer up to the circular carbon webbing 2nd layer over anchor arms |
| Perlon Stockinette | 623T3=* | 2 | completely |

# SF5R, SF6A up to 125 kg

| Preparation Step |  |  |  |  |
| --- | --- | --- | --- | --- |
| Material | Kenn-zeichen | Lagen Anzahl |  | Kommentar |
| Adapter |  |  |  | aufsetzen |
| Dakron Felt | 616G6=* | 1,5 |  | 1x completely and 2nd layer proximal socket brim |
| Carbon Fiber webbing - unidirectional | 616B1=50X | 2 |  | width 5 cm circular 3 cm below perineum |
| Woven Carbon Fiber Stockinette | 616G15=* | 1,5 |  | komplett Abbinden unter Vierkant |
| Carbon Fiber webbing - unidirectional | 616B1=50X | 4 |  | as V in ischial tuberosity seat |
| Carbon Fiber Cloth | 616G12=* | 3 |  | 15x15 cm distal stump end slightly shifted |
| Perlon Stockinette | 623T3=* | 2 |  | completely |
| Final Step |  |  |  |  |
| Perlon Stockinette | 623T3=* | 1 | completely |  |  |
| Carbon Fiber Cloth | 616G12=* | 2 | 15x15 cm distal stump end slightly shifted |  |  |
| Woven Carbon Fiber Stockinette | 616G15 | 1,5 | First layer up to the circular carbon webbing 2nd layer over anchor arms |  |  |
| Perlon Stockinette | 623T3=* | 2 | completely |  |  |